# Regulations 2026

Endurance AutoCross Belgium asbl

General regulations.

1. Participating pilots;

What to do when you arrive at the site:

- 1) Find a place at the riders' park.

- 2) Inspection of the competition car. Saturday morning from 7:30 am at the EACB inspection tent. If you are in possession of an annual inspection, you can skip this step. Our inspection tent is usually at the start. Recognizable by the flag(s). You can combine this with having your transponder installed.

- 3) Pick up the transponder and set the bib number. You also do this at the inspection tent. You must hand over your passport as a guarantee for your transponder. Come with your car to our tent. Our people will install the transponder on your car.

- 4) Insurance. Take out mandatory pilot insurance in the big tent. If you have an annual insurance, you can go directly to the registration.

- 5) Registration. Fill out a registration form and bring it to the registration table.

- 6) Check the placement of the transponder yourself. In case of loss, you pay 70€ as compensation.

- 7) You are now ready to go. Come to the start with your car 30 minutes before your start time and follow the instructions of the EACB staff for the brake test.

- 8) 10 minutes before the start of the 1st part, a briefing is mandatory for all participating pilots.

- 9) End of the match. Take the transponder from your car and return it to the EACB tent. You will receive your passport back when you hand in your transponder.

What are the costs:

- The entrance costs 20€ per person for 2 days or 15€ for one day. Up to 12 years free.

- The inspection of your car costs 2€ for a day inspection or 5€ for an annual inspection.

- Your insurance as a pilot costs 15€/day or 35€/year.

- Registration costs 70€ per car in the S, S+, A, A+ and X class. For the youth J and J+ you pay 30€ and the mini buggies pay 10€.

- Self-adhesive numbers and backgrounds are available at the EACB tent at the price of 1€/piece.

- Each participating adult pilot must  report to the organizer in possession of his driving license or his provisional driving license and register. Registration forms can be found on our site www.eacbautocross.be on the "documents" page or can be obtained from the organizer.

- Youth pilots come with their passport and are registered by his/her parent(s) or guardian. By signing the registration form, he declares that he has the permission of his fellow family members and/or the parent or guardian who is not present to allow the young pilot to drive. The signing parent or guardian assumes all the responsibilities of the "first pilot" in these rules. Registration form can be obtained from the organizer. Registration for youth is possible until 8:45 am. First start youth is at 9 am!!

- By signing the registration form, the pilot declares that he has taken note of the general regulations and the technical regulations intended for his class and that he complies with them. Any incorrect information on the registration form is the responsibility of the first pilot. Incorrect data with the intention of fraud will result in a sanction.

- Only registered pilots who have attended the briefing of their series/category are allowed to participate in the competition! In the event of a breach, the first pilot will be declared in default and will bear all responsibility.

- It is the "first pilot" who is considered the responsible person of the team. Only he or she gets the points of the championship to his or her name. Also under his responsibility, any damage caused to "official" vehicles and machines before, during and after the competition.

- It is strictly forbidden to participate in the competition under the influence of "alcohol/drugs". Preventive alcohol checks will take place in random series. The pilots who have to blow will be determined by lottery. The organizer and the race management always have the right to let pilots blow if they suspect that they have looked too deeply into the glass. The limit used is 0.2 per mille. Control may be possible with exclusion for the entire team as a result of a possible violation.

- Wearing a helmet, long sleeves, long pants, motocross goggles, neck protection and a fixed belt is mandatory during the inspection, competition and any practice round.

- Environment! Respect greenery & nature as well as the guidelines of the local designer! Also the roads to it and the neighborhood. Fleet is not a test track.

- Every participating pilot must take out personal insurance. The price of a day or annual insurance is according to the rate of NVACT. This insurance only counts for competitions that are ridden together with NVACT in Belgium.

- All pilots entering the circuit must wear a wristband provided for this purpose. The organizer, possibly in consultation with the inspectors, checks this every time a car enters the circuit. If fraud is found with the wristband or pilot bracelet, the car with all registered pilots will be suspended from participation in all races of Endurance Autocross Belgium vzw.

- Should a non-registered pilot nevertheless enter the course and cause damage and/or personal injury, the organizer and the official of Endurance Autocross Belgium vzw reject all responsibility.

2. Participating cars;

- All cars must be presented for inspection before proceeding to registration. It is possible to take a day or annual inspection. The car must be presented for inspection in good condition. You need the inspection certificate for your registration and for the brake test which is done for your first series.

- All registered cars must bear a clear participation number assigned by EACB. The number must be on a background of about 20x25 cm with numbers of minus 15x12cm. Be CLEARLY legible (roof number plate). The background of this board must be the right color. Self-adhesive backgrounds and numbers are available at the EACB tent.

- Each car counting for the final result must be equipped with a transponder (to be collected after registration under guarantee) corresponding to the registered number. Hand over a pass or driver's license as a deposit.

- A mesh and/or net on the door of the driver's side and half the front of the windshield (driver's side) is mandatory.

- No original seat belt, but at least a decent 4-point seat belt is mandatory. This belt must be firmly attached to the chassis of the car. Not to the wooden beams that have been placed, for example.

- All windows, lights (original), exterior mirrors, interior trim, trim/bumpers, as well as any towbar with FRAME must be removed.

- Plastic bumpers should preferably be removed. However, for cars that are also used for sprint series, they can remain if they are extra solidly attached. The inside of the bumper must then be visible and verifiable.

- LPG installations are prohibited and must be completely removed!

- Each car must be equipped with a clearly marked towing point at the front and rear at the bottom. Not protruding. Under no circumstances should it go past the bumper. Use a chain in a loop. The commercially available ribbons are beautiful but not strong enough!!

- The pilot seat must be properly attached and equipped with a headrest (original or not)

- All cooling tubes to and from the radiator that run through the driver's compartment must be insulated or shielded so that direct contact by humans is not possible.

- The battery must be properly (extra) fixed and shielded. The PLUS born MUST be insulated!

- For the youth categories, A, A+ and X class, the car must be equipped with an externally operated circuit breaker. This should be on the right side of the car and easily accessible from outside the car. The cart must stop when the circuit breaker is operated.

- For the S and S+ class  , a circuit breaker may be installed but it is not mandatory.

- A non-return valve on the ventilation of the fuel tank is mandatory.

- The cars may not have any protruding sharp or loose parts at the start of the race.

- All rear drive cars must be equipped with (decent) mud flaps 3/2 of the width of the tire and MAX 15cm from the ground!

- Side doors or, depending on the class, replacement plates are mandatory! (min 2/3 of the door height). Any open roof must be covered with welded plate.

- Only for the S Class and the S+ Class is a minimum of three metal construction dots between roof and floor allowed, 1 at the front next to the pilot and 1 behind the driver's seat and 1 on the passenger side parallel to the one behind the driver's seat. All screwed with at least 3 bolts M10 in the bottom and 3 bolts M10 in the roof. A roll cage with extra protection inside the driver's door is highly recommended!!

- All cars of the youth categories, A, A+ and X class must be equipped with a roll cage (min 40mm), properly attached at minus 4-6 points on the chassis (bottom). A plate of at least 3 mm thick and 10 by 10 cm in size must be welded between pipe and bottom. The roll cage must surround the driver's seat. Additional protection on the inside of the driver's side is strongly recommended!

- Each participating car must be equipped with a properly functioning braking system (4 wheels lock!)

- Each participating car MUST be equipped with 1, preferably 2 continuously burning dust/mode (anti-crash) lights on the back of the car and MINUS 1, preferably 2 traffic light(s) on the back of the car. All type of fog light minus 21W (red!) and separated from each other. LED lights are allowed if they contain at least 40 LEDs. If your lighting does not work during the match, there is a chance that you will be pulled over to repair it.

- Cars that do not comply with the technical regulations intended for their registered category will not be allowed to participate in the competition within the championship.

- Electric cars are not allowed.

TIRES:

On the towing wheels one may drive with:

- Eurocross tires or similar. The maximum permitted dimensions of the cross cuts are 14mm wide and 14mm deep. These are the grooves that go from the inside to the outside of the tire. The grooves in direction, around the band around are free.

- Also all common street/snow tires (M/S) whether or not cut out with a maximum depth of 6mm. Width cutouts unlimited. This is true for all categories. In bad weather conditions, after consultation with the race directors and organizers, a different type of tire can exceptionally be allowed per category/series.

3- Categories;

S class;

- Complete original car (no 4x4, lowered or widened beetles, self-built, etc.).

- All cars must meet all the points described in the chapter "Participating cars". Only the placement of a chalice is allowed. Furthermore, no form of tuning is  allowed!!

- The exhaust must be solid and extra fixed and end at the back or left and may be placed inside, but according to the rules of the art and with the materials used for this purpose. Not at the bottom! MAX 90db, also during the race!!

- The roof license plate must have a white background with black numbers.

- Bodywork, engine (original carburetion/injection), entire drivetrain with gearbox and suspension remains original, brand and type bound! The car must have existed originally, engine versus bodywork. No limited slip differential and or reinforced (claw shifted) gearbox, even though it is original!

- Bodywork must not be shortened. Doors may be welded shut, either with MAX 3 to 5 welds of 5 cm each. The trunk lid must be present and may only be welded shut at 2 points (weld seam of 5cm), the interior space (trunk space) must be fully visible

- All components remain in the original place! (Battery, radiator, air filter, etc., properly recorded)

- No reinforcement or reinforcement of the bodywork is permitted, with the exception of the roll cage (as described in chapter "Participating cars"). This may be connected to the chassis at a maximum of 6 points within the suspension points (suspension/shock absorbers) of the car. Reinforcements on the outside of the car will no longer be allowed from 2027.

- These cars may have a crankcase protection! Focused on the environment and not to strengthen the car. Not connected to front bumper.

- All plastic inner mudguards must be removed!

- Plastic bumpers should be removed. The front and rear bumpers, if removed, may be replaced in the original place by a square or round tube. (Max 2mm thick and a diameter of max 50mm) The ends of the bumpers must be rounded or slanted (no sharp corners) and must remain 10 cm within the width of the body on both sides. The combination of tube and plastic bumper is NOT allowed! If the car is used in the sprint series, the plastic bumper may remain in place, but it must be controllable on the inside and be extra fastened.

- Rims may not be reinforced.

- Every car must have a properly functioning braking system!!

S+ Class;

- Original car (no lowered or widened beetles, 4x4, self-build, etc.).

- The roof license plate must have a green background with white numbers.

- All cars must comply with the chapter "Participating cars".

- Bodywork, engine (original carburetion/injection) and drivetrain with gearbox remain original, brand, type and location! The car must have existed originally, engine versus bodywork. No limited slip differential and/or reinforced (claw shifted) gearbox. Even if it is original!

- The bodywork must not be shortened! Only the doors may be completely welded shut. The trunk lid must be present and may only be welded shut at 2 points (weld seam of 5 cm), the interior space (trunk space) must be fully perceptible. The spare wheel cover may be replaced by a thin plate. (Not completely welded, max 5 welds of 5cm)

- The original suspension/suspension remains, but only the wishbone pivot points and springs may be provided. (No "Proflex", etc. Shock absorber/ spring combinations)

- Radiator, battery, exhaust and petrol tank (pipes) may be placed inside according to the rules of the art. Be properly fixed, shielded and made in the materials provided.

- The exhaust max.90db, (also during the race) must be firmly fixed and flow out at the back or side left (not at the bottom)!

- These cars may have a crankcase protection! Focused on the environment and not to strengthen the car. Not connected to front bumper.

- All plastic inner mudguards must be removed.

- The front and rear bumpers, if removed, may be replaced in the original place by a square or round tube. (Max 2mm thick and a diameter of max 50mm) The ends of the bumpers must  be rounded or slanted (no sharp corners) and must  remain 10cm within the width of the bodywork  on both sides.The combination of tube and plastic bumper is NOT allowed! If the car is used in the sprint series, the plastic bumper may remain in place, but it must be controllable on the inside and be extra fastened.

- No reinforcement or reinforcement of the bodywork is permitted, with the exception of the roll cage (as described in chapter "Participating cars"). This may be connected to the chassis at several points within the suspension points (suspension/shock absorbers) of the car. Reinforcements on the outside of the car will no longer be allowed from 2027.

- Limited tuning allowed! (Inlet and outlet only) - (chalice or spaghetti)

- An adjustment of the engine management is allowed (such as KMS, DTA, LINK ...).

- Replacing the ECU is allowed.

- Rims may be reinforced.

- Every car must have a properly functioning braking system!!

A Class;

- All cars must meet the points described in the chapter "Participating cars"

- The roof license plate should have a yellow background with black numbers.

- No 4x4, self-build, lowered/widened beetle, etc. are allowed in this category.

- Cars must be built from a commercially existing model/type. The original underbody must be retained above the rear axle. Only the spare wheel cover may be removed.

- Bodywork and engine (max. 2000cc) are brand and place specific.

- No turbos or compressors or rotary engines allowed! (Petrol and diesel)

- The original suspension, position in relation to bodywork remains, but only the springs and shock absorbers as well as the wishbone pivot points may be replaced/strengthened or raised. This allows eye/eye shock absorbers to be placed on the non-steering wheels. Spring combinations such as Proflex or the like are allowed. No self-designed suspension (wishbones, etc.)

- All mechanical component battery, exhaust, pipes, etc.) that are moved and/or replaced must be properly fixed and executed according to the rules of the art with the materials used for this purpose. Cooling system & exhaust must be shielded from the pilot and competitors.

- The exhaust max 90db, (also during the race!) must be properly fixed and end at the back or side left (not at the bottom)!

- Any form of tuning (engine and drivetrain) with the exception of turbo or compressor, original or not are allowed.

- To a limited extent, any reinforcement and/or reinforcement of these cars is permitted.

- (No "Bangers or Mad-Max" cars !!)

- For the rear tractors, mud flaps are mandatory up to a maximum of 15cm from the ground!! (On the driven wheels)

- The cars must not have sharp protrusions. Bumpers must have rounded & closed ends.

- Every car must have a properly functioning braking system!!

A+ Class;

- All cars must meet the points described in the chapter "Participating cars".!!

- The roof license plate must have a red background with white numbers.

- Cars must be built from a commercially existing model/type. The original bottom plate (chassis) must be retained for 2/3.

- No self-builders (sprinters), beetles with motorcycle engines, SSVs or complete tubular frames.

- Beetles or not fitted with a car engine whether or not VW are allowed.

- 4x4 passenger cars are allowed Original or not.

- The engine is not brand-specific. No cc restriction!!  No motorcycle engines.

- All mechanical components (radiator, battery, exhaust, pipes, etc.) that are to be moved and/or replaced must be properly fixed and made in accordance with the rules of the art with the appropriate materials! Cooling system & exhaust must be shielded from the pilot and competitors.

- The exhaust max 90db, (also during the race) must be properly fixed and flow out at the back or side-left (not at the bottom)!

- For the rear tractors, mud flaps are mandatory up to a maximum of 15cm from the ground!! (On the driven wheels)

- Any kind of tuning and reinforcement as in cat. A are allowed (free).

- The suspension (suspension, suspension principle, shock absorbers, etc.) may be modified, moved, reinforced and/or replaced. (Free choice)

- The cars must not have sharp protrusions. Bumpers must have rounded & closed ends.

- Every car must have a properly functioning braking system!!

X Class;

- The roof license plate must have a blue background with white numbers.

- Each car must comply with the points described in the chapter "Participating cars", with the difference that these cars  must not be built from a commercially existing model/type of body.

- For all; cars, beetles, self-built 4x4, platforms, SSVs etc., the engine (including motorcycle motorcycles) and the bodywork/frame, are not brand and place bound!! (No jeeps)

- All mechanical components (radiator, battery, exhaust, fuel tank, pipes, steering, engine, etc.) that are to be moved and/or replaced must be properly fixed and carried out in accordance with the "rules of the art" with the appropriate materials. Cooling system & exhaust must be shielded from the pilot and competitors.

- The exhaust max 90db, (also during the race) must be properly fixed and flow out at the back or side-left (not at the bottom)!

- Any form of tuning and reinforcement is allowed. (Free) No "Bangers" or "Mad-Max situations"!!

- The complete suspension may be modified, moved, strengthened and/or replaced.

- The cars must not have sharp protrusions.

- Every car must have a properly functioning braking system!

- For the rear tractors, mud flaps are mandatory up to a maximum of 15cm from the ground!!

J class (Youth);

- Youth pilots from 12 to 15-year-olds may participate from the day they turn 12 years old  up to and including the year they turn or have become 15 years old.

- They must have permission from either parent or guardian to participate.

- The youth pilots must have a sufficiently safe steering skill before they participate in our competitions. If the race management or organizer decides that this is not the case, they must immediately stop the race or they may not start.

- All cars must comply with all the points described in the chapter "Participating cars" of the general regulations.

- An adjustment of the engine management is allowed (such as KMS, DTA, LINK ...).

- Replacing the ECU is allowed.

- The gearbox, without intermediate plate, must belong to the make of the car and must remain original. 4 x 4 is not allowed.

- The engine block and crankshaft must remain completely original.

- The engine, throttle valves, injectors must remain original. A spaghetti outlet is allowed.

- The suspension and shock absorbers must be original and may not be modified.

- The pilot or companion must be able to point out the engine code during the inspection and it must be legible. Also during interim checks.

- A roll cage and external circuit breaker is mandatory for the youth (no beams allowed).

- The maximum cylinder capacity is 1400cc.

- Cars with a supercharging system (turbo, compressor, etc.) and rotary engines are not allowed.

- The roof license plate must have a purple background with white numbers.

J+ class (Youth);

- Youth pilots from 15 to 18-year-olds may participate from the year they turn 16 up  to and including the year they turn or have become 18 years old.

- They must have permission from either parent or guardian to participate.

- The youth pilots must have a sufficiently safe steering skill before they participate in our competitions. If the race management or organizer decides that this is not the case, they must immediately stop the race or they may not start.

- All cars must comply with all the points described in the chapter "Participating cars" of the general regulations.

- An adjustment of the engine management is allowed (such as KMS, DTA, LINK ...).

- Replacing the ECU is allowed.

- The gearbox, without intermediate plate, must belong to the make of the car and must remain original. 4 x 4 is not allowed.

- The engine block and crankshaft must remain completely original.

- The engine, throttle valves, injectors must remain original. A spaghetti outlet is allowed.

- Cars with a supercharging system (turbo, compressor, etc.) and rotary engines are not allowed.

- The suspension and shock absorbers must be original and may not be modified.

- The pilot or companion must be able to point out the engine code during the inspection and it must be legible. Also during interim checks.

- A roll cage and external circuit breaker is mandatory for the youth (no beams allowed).

- The maximum cylinder capacity is 1400cc.

- The roof license plate must have an orange background with black numbers.

4- Rennerspark;

- Each first registered driver of a car is responsible for his location as well as for the behaviour of supporters, mechanics and friends accompanying the team.

- Take your guying equipment (ribbon, nameplate, pickets, etc.) back with you afterwards! Do not leave pickets or tent pegs in the ground!

- The responsible driver is liable for any possible accident caused by the team car in the riders' park and/or circuit.

- Competition directors and organizers will monitor this and take the necessary sanction if necessary.

- A waterproof groundsheet must be placed under each participating motocross car  , even if it  is brought the day before!

- Tinkering, refueling, etc. is only done at the designated pitch and on the groundsheet! (supervision is provided by the organization)

- No  driving test of the car may take place in the fleet.

- The MAX speed is 5km/h in the fleet!

- When registering, a deposit may be requested for the location. This must be left in good condition (no traces of oil, parts, glass and/or litter). After checking the organization, the deposit can be refunded.

- There will be no waste container. You take your own waste home with you.

- A properly functioning fire extinguisher (min 6kg) is mandatory for every car.

- The deposit will only be paid half an hour after flagging off your class!

5- Technical inspection;

- The technical inspection starts every race day at 07.30 in the morning.  It is mandatory to have a day or annual inspection done. You will receive an inspection card which you need to register for the competition(s). For an inspection you can offer your car from 7.30 am regardless of the class in which you want to register your car, although it would be useful if cars for the A, A+ and X class would only come from 9.20 am.

- For the youth categories, the engine code must be identifiable and legible. These will be checked.

- The brake test will take place before the start of the first part of the race.

- Be on time!! Ten minutes before the start of a series, no more inspections are done for the starting series!!

- Cars that exceed the technical regulations of their category are placed one category higher or out of the championship without any discussion.

- If a comment is made or if the car is rejected, the team gets the chance to make the car compliant (in the riders' park!) and to offer it again at the inspection.

- Once the starting position has been taken, it is no longer allowed to  tinker or change places. Only with the permission of the official EACB can anything be done.

- After the flag has been flagged, the EACB reserves the right to pull over some participating cars, either arbitrarily or according to certain structures, for an unannounced check of the driver and car. If things are found that are not according to the rules, his result will be deleted, including his starting points. Checks will be done!!

- There is no recourse against the decisions of the organisation, race director and race commissioners. The aforementioned cannot be held liable for any accidents, theft and/or damage to vehicles and personal belongings.

6. Competition & Championship;

- Daily schedule:

07:30 start of the judging. Youth until 8:40 am, S and S+ until 9:25 am, the others until 11:00 am. Registration is from 07:30 to 11:00.

- 09h - 9:40h Part 1 series J and J+ (Youth)

- 9:50 am - 11:20 am Part 1 series S and S+

- 11:30 am - 11:45 am Part 1 series Mini Buggy

- 11:55 am - 1:25 pm Part 1 series A, A+ and X

- Break

- 13:55h - 14:35h Part 2 series J and J+ (Youth)

- 14:45h - 16:15h Part 2 series S and S+

- 16:25h - 16:40h Part 2 series Mini buggy

- 16:50h - 18:20h Part 2 series A, A+ and X

The daily schedule can be adjusted unannounced if necessary. E.g. spraying extra water etc... The goal remains to finish by 7 pm. (As long as daylight allows.)

- Scoring for all categories. It is only the first driver who collects points for a car. Keep this in mind when registering!! This cannot be changed retrospectively under any circumstances. Each registered first pilot gets 5 starting points. These starting points increase according to the number of races started.

- First started match 5 points

- Second game started 6 points

- Third game started 7 points

- Etc...

The match points to be earned based on the final result will be distributed as follows

- Winner 20 points

- 2nd place 18 points

- 3rd place 16 points

- 4th place 14 points

- 5th place 12 points

- 6th place 10 points

- 7th place 9 points

- 8th place 8 points

- 9th place 7 points

- 10th place 6 points

- 11th place 5 points

- 12th place 4 points

- 13th place 3 points

- 14th place 2 points

- 15th place 1 point

- For the championship, all these points are added together. The person with the highest number of points can call himself the champion. In the event of an equal number of points,  the total number of laps driven in all races will be decisive. Then most of the 1st place, then the 2nd places, etc

- Race directors and safety people will use flags to make certain things clear to you as a pilot, during and after the race. Make sure you know and respect the meaning. Failure to comply with these orders may result in a sanction ranging from "stop & go" to exclusion.

- The pilots are bound by the instructions of the race director, the track marshals and the safety people.

- In the event of a breakdown, every pilot remains in his car unless the security people signal him to get out or if his safety is seriously endangered (e.g. in the event of a fire).

- When towing away, the indicated exits in the direction of travel are used and the pilot concerned keeps his helmet on! (Follow the instructions of the security people)

- Being pushed further (over the finish line) in case of breakdown by a fellow contender or by a tractor/jeep of the organization does not give an extra lap.

- Driver change will only take place  at a designated place and/or location (riders' park). Under "Red" no one rides "up or down" the course.

- Vehicles of the towing service, spray tractor and vehicles of the organization always have priority and must be passed on the widest side, they are in themselves a yellow-flag (see explanation) situation. Any damage to the aforementioned vehicles due to collision is at the expense of the responsible pilot ("1st pilot") of the approaching car.

- Track marshals and race director have the last word.

7. Meaning of the flags used;

The track marshals have different flags. These flags have the following meaning:

- Yellow flag

  - Yellow moved: Indicates that a dangerous situation has arisen. During a yellow flag, overtaking is allowed, but this must be done in a sporting manner and with the necessary caution . The instructions of the track marshals must be followed.

  - Yellow NOT moved: indicates that there is an obstacle on the track (e.g. a kilver board, tow truck, ...). The instructions of the track marshals must be followed.

- Red flag

  - The match is stopped because a dangerous situation has arisen. You must stop in a safe way (DO NOT stop abruptly).

  - It is not acceptable that you pass several track marshals under a red flag.

  - Ignoring the red flag will automatically lead to a black flag or to the reversal of the car in question to where the race director thinks it is necessary!!

- Blue flag

  - At a restart, the track marshals raise their blue flag. This is an indication to the race director that the track is free and that it is possible to start.

- Black flag

  - This can be given for various reasons (unsportsmanlike driving, technical black, ...).

  - You have to stop at the race director and you will receive a short explanation about the violation committed and then the corresponding sanction. Afterwards, you can continue your match.

  - If you receive this black flag several times for the same facts, you may be permanently excluded.

- Flag half black, half white.

  - This is a warning flag. If you get to see this flag, you don't have to stop and you can continue your race and ask for information later.

  - If you get this flag 2x for the same, this will lead to a black flag!

- Black flag with orange ball:

  - Technical black flag. This flag is shown, NEVER waved.

- Belgian flag

  - Starting flag

- Black and white checkered flag (finish flag)

  - End of the match.

8. Start procedure.

- Starting places from 1 to 20 are provided by means of numbered signs which are placed in two rows (pair - unpair).

- Preparing for the start of your first series goes as follows:

- The first start of the first race of the year, the places will be taken according to the final standings of the previous year's championship. This means that the pilot who finished 5th last year, for example, places his car at sign 5. New drivers within the category and those who finished outside the selected places join after twentieth place. (From the sign "+20")

- Of the S and J class, the first 10 place on the odd side and the first ten of the S+ and the J+ class place on the odd side. This is only for the first race of the year!!

- The A, A+ and X class work differently. Here we work with 21 places, 7 for each category. Last year's three champions will be in the first three places. The champion A class sign 1, champion A+ class sign 2 and Champion X class at sign 3. Then at sign 4 the 2nd from the A class, sign 5 the 2nd A+ car, sign 6 the 2nd from the X class etc. until the 21 places are full. This is only for the first race of the year!!

- From the second start and all subsequent competitions, the start will be made according to the general result of the previous series or competition. This means that the fifth in the overall result qualifies for board 5 regardless of his class. The general results will be available at the EACB tent.

- Preparing for the start of your second series goes for the first twenty according to the general result of the first series. The others (+ 20) simply join from the "+20" sign.

- The starting places will be checked.

- First lap we are still behind the starting car and we are not allowed to overtake before the starting flag is waved!! Unless in the event of a breakdown in the previous car. There will be a check on this!! At the start, one can queue up to his predecessor. Example: starting place five is not occupied, then the car in seventh place may join the car in third place and so on....

- This starting procedure can exceptionally be deviated from in special (non-standard) crosses. This will be communicated in good time.

9. Procedure restarted after a red flag.

- The track marshals raise their blue flag. This indicates that the track is safe and that the race director can give the start safely.

- The race director uses the Belgian flag to give the restart. The track marshals use the blue flag to give the start.

- Flags down = Start

10. Some important rules, agreements and sanctions.

- Every penalty that is given counts for the car (i.e. the entire team).

- The drivers' briefing is mandatory for all drivers.

- In the paddock you drive at walking pace (= 5 km/h).

- If you are caught not complying with this rule:

- 1st time: 5 penalty rounds;

- 2nd time: 10 penalty rounds;

- 3rd time: definitively excluded.

- The dust lamp of your car must be on for the entire race. If this does not work, this will be communicated to the inspection by the race director.

- Ignoring track marshals is punished in the following ways:

- 1st time: 5 penalty rounds;

- 2nd time: 10 penalty rounds;

- 3rd time: definitively excluded.

- The deliberate driving of other cars will be punished immediately:

- 1st time: 10 penalty rounds

- 2nd time: excluded for that particular race/heat (for the multi-hour race you can be excluded for a certain time).

- From the moment the car is on the course, the pilot must ALWAYS wear his helmet.

- It is not allowed to leave the car during the races (not even under a red flag situation), unless otherwise indicated by the track marshals or in the event of a fire.

- It is strictly forbidden to ride with short sleeves and/or shorts.

- Wearing neck protection is mandatory.

- Spectators, mechanics, ... are NOT transported in the dirt car!

- For spectators, mechanics, supporters, ... it is STRICTLY FORBIDDEN to enter the course during the races. If a serious incident has occurred, a maximum of 1 person may enter the course, after permission from the race director.

- The pilot(s) are responsible for their entourage (supporters, mechanics, etc.) during the entire race day. If they do not comply with these rules, fight, show rude behavior towards the track marshals, jury, organization or the race director, the team will be sanctioned.

- Only in case of technical breakdown before the start, the registration fee will be refunded in consultation with the organization (deduction of €15 administration). Once started and the count has passed,  the aforementioned right lapses.

11. Reglementen Mini buggy's EACB

- The Mini buggies drive a full-fledged championship like all other categories.

1. General

- In this class, only vehicles drive single-seater frames with bodies specially designed for autocross and whose silhouette bears no resemblance to a mass-produced car.

2. Pilots and escorts

- 2.1. Participation in this class is possible from the age of 6. From the year that the pilot turns 12 years old, that year may be completed even if the pilot is 12 years old at the first cross.

- 2.2. The parents and/or guardian (supervisors) MUST provide guidance up to and including the pre-start. After this, they have to go behind the fence.

3. Safety construction

- 3.1. The following drawings show the minimum pipe thickness gauges and shape:

- 3.2. All pipes must be fully welded.

- 3.3. The roof sheet shall be made of steel. The roof sheet must have a minimum thickness of 1.5 mm. The roof plate must be welded to the 2 main brackets and the cross-connections welded between them.

- 3.4. The base plate shall be made of steel and have a thickness of at least 1,5 mm. The bottom plate must be properly welded from the pedal tube A to crossbar B. A metal plate must be mounted from the crossbar tube B with a proper weld or lip/screw connection to the crossbar tube C (steel thickness 1.5 mm / metal thickness 2.0 mm). The base plate must be completely closed from the pedal tube A to the cross-connection tube C.

- 3.5. The front window must be fitted with mesh at least 2 mm thick and with meshes of no more than 40x40 mm or 50x25 mm.

- 3.6. A mesh with holes of up to 40x40 mm or 50x25 mm shall be fitted on the sides. This mesh must be mounted with 2 blocked hinges and a proper closure, which must also be open from the outside. The mesh must shield the entire side entrance.

- 3.7. Wheelbase: maximum 1500 mm (measured on the centre of the wheel) 3.8. Front track: maximum 1200 mm (measured outside of tyre). Rear track: maximum 1300 mm (measured outside of tyre).

4. Safety/seat/seat belt

- 4.1. A well-fitting seat with a headrest is mandatory.

- 4.2. A Polyester/composite chair must be equipped with a steel chair frame, thickness of the tube must be min. 15x1.5 mm.

- 4.3. The chair must be properly attached at 5 points.

- 4.4. At least a 5-point safety belt with, safety buckle with hips, shoulder adjustment with hook attachment and eyebolts. The belt must comply with the ECE/EEC standard. The fifth point must be attached to the cross connection B of the roll cage. The size of this pipe (cross connection B) must be at least 22 x 1.5 mm. All other belt tips must be attached to a tube that complies with the minimum size of 25 x 3 or 30 x 2 mm along its entire length. The backstage belt guide tube must meet a minimum size of 25 x 3 mm over the entire length. (belt eyes must be properly welded). The backstage belts may be looped around a tube of 25 x 3 / 30 x 2 mm.

- 4.5. The mandatory main circuit breaker must be capable of being operated from both the inside and the outside. As far as the outside is concerned, the control mechanism must be attached to the underside of the windscreen/mesh

- 4.6. A working clearly visible dust lamp is mandatory (no reflector).

- 4.7. A brake light is recommended.

- 4.8. The fuel tank shall be shielded from the driver's compartment by means of an aluminium plate of at least 1,5 mm. The engine compartment shall be completely separated from the operator compartment by a metal closed wall of at least 1,5 mm thickness.

5. Personal equipment

- 5.1. A well-fitting undamaged helmet with a good closure is mandatory, with visor or goggles.

- 5.2. A race or cross overall is mandatory.

- 5.3. A neckband is mandatory. The neckband must be in good condition. Also, the neckband should fit snugly between the helmet and the rider's shoulder.

- 5.4. Appropriate clothing: such as gloves and good footwear are recommended.

6. The Contest Number

- 6.1. The bib number must be clearly visible on 2 sides by means of a white background and black numbers with a minimum height of 140 mm x width of 80 mm.

7. Motor and drive

- 7.1. Engine must be: Honda gx200 or a Loncin ptm200.

- 7.2. Both mounted with the original tank 3,6L, which must be mounted on the original points.

- 7.3. Standard exhaust (no modifications allowed).

- 7.4. Air filter is free, but the base plate No. 8 must be retained (see drawing below). If the crankcase ventilation is removed from the light filter, this must be collected in a catch tank. Base plate 8 may have a maximum internal passage of 30 mm.

- 7.5. Original carburettor (no modifications allowed).

- 7.6. The rev limiter may be removed and therefore tighter valve springs may be fitted with part number: 14751-ZE1-000 of a Honda gx140. For the rest, the engine must remain original!

- 7.7. The drive to the rear axle is provided by a wet centrifugal clutch with a 2:1 reduction (PTM engines).

8. Brake system/rear axle/shock absorbers

- 8.1. A properly functioning hydraulic brake on the rear axle only. This will be statically checked before the start of the first part.

- 8.2. The rear axle, at least 30 mm, is fitted with universal joints or CV joints and no differential may be fitted.

- 8.3. The same shock absorbers are used all around. Type: RFY QR-280 /300.

9. The Gang/Velgen/Spades

- 9.1. Choose: steel choose.

- 9.2. Rim size for 8 inches and maximum width of 160 mm. Rear rim size 8 inches and maximum width of 170 mm.

- 9.3. Mudguards on the rear wheels are mandatory and must be properly fastened. The mudguard should cover 50% of the entire wheel. The fender should be made of PE material with a thickness of 2.0mm.

- 9.4. The minimum weight of the Participating Vehicle (excluding the Rider and his complete Driver Equipment) including all liquids still in the Participating Vehicle at the time of weighing must be at least 130 kg.

10. General info

- The Mini buggies run twice for 15 minutes and start at 11:30 am for their first series and at 4:25 pm for their second series.

- Registration 10€.  Pilot insurance is also mandatory at the normal price.

- Inspection is done when preparing for the start. The points of attention are: the brakes, the seat belts tightened, the helmet fastened and general safety.

Endurance Autocross Belgium wishes you a safe and fun race!!